T2081 custom board

Regarding our board from JTAG we have TCK, TMS, TDA, TDI is connected to t2081 processor and Soft reset, hard reset and Target reset is connected to FPGA.

The error I am getting is failed to correctly  configure the jtag chain

Running Hello world program:

We are running the program using codewarrior tap and codewarrior ide as follows

I have also attached the steps on how I am creating rcw below after few steps.

I am getting the same error even I initialize by using sram, ram, download, connect etc.

This is the error image.

Steps used for creation of default rcw hardcoded options

Also I am getting the same error when creating rcw from t2080rm.pdf 4.6.4.1.2

DESIGN OF THE BOARD

NOR FLASH1

256MB

FPGA

T2081

PROCESSOR

NOR FLASH2

256MB

IFC bus                                               Local bus

Multiplexed                                       Demultiplexed

MRAM

256KB

The above picture shows the block level connection between processor and FPGA via multiplexed

The resets from the JTAG are connected to the FPGA .

The logic is

assign proc_jtag_trst = ((flag == 1'b1) ? PORESET : 1'b1  )  & req_rst_r   &  proc_jtag_trst_n;

assign  HRESET_N = ((PORESET) ? req_rst_r : 1'b1) ? 1'b1 :  1'b0;

assign  {IFC_AD8 ,IFC_AD9 ,IFC_AD10 ,IFC_AD11 ,IFC_AD12 ,IFC_AD13 ,IFC_AD14 ,IFC_AD15 ,IFC_CLE} =  ((req_rst_r & PORESET)== 'b0) ? rcw_hardcoded :  9'bz   ; // hardcoded

assign  {IFC_AD7,IFC_AD6,IFC_AD5,IFC_AD4,IFC_AD3,IFC_AD2,IFC_AD1,IFC_AD0} =  ((req_rst_r & PORESET)== 'b0) ?  8'hff :  8'bz   ;

The below picture discloses the output for reset logic implemented in the FPGA.