Hi NXP Team,

We are currently following the Building SPL and bootloader images with encrypted boot enabled procedure on the i.MX8MPlus platform with Android 14 (L6.1.55_2.2.0), as outlined in the i.MX Android User’s Guide.

As part of this process, we have some doubts for which we want the clarification.

1.) In Android User’s Guide

In section 7.1.6.1 Building SPL and bootloader images with encrypted boot enabled

It is mentioned “see Section "Building Android images to construct the containers" and Section "Enabling the encrypted boot support in U-Boot" in the i.MX Android Security User's Guide (ASUG) to enable the encrypted boot function by modifying the target defconfig files”

Here our opinion is we need to follow only section “Enabling the encrypted boot support in U-Boot” for our platform. Is it correct?

2.) In Android User’s Guide

In section 7.1.6.2 Encrypting SPL and bootloader images

It is mentioned “To facilitate remote upgrades, all the CST commands that encrypt images should be appended with the -d parameter. This parameter requires CST to reuse DEK Blob files that already exist in the current directory”

Can you clarify how to use this parameter?

3.) How to verify our bootloader is single-bootloader or dual-bootloader?

4.) In  i.MX Android Security User's Guide

In section 3.1.4.8 Creating the final CSF binary for SPL image

Calculate the Nonce/MAC size based on the MAC bytes value in CSF:

Nonlculatece/MAC size = Nonce size + MAC bytes + CSF header for Nonce/Mac = 12 + 16 + 8 = 36 bytes

Are these values are constant or we need to modify? If to modify means, how to get these values?

5.) In  i.MX Android Security User's Guide

If we follow the “Encrypted boot with HABv4” means we need to do the steps of

3.1.4.6 Creating the CSF description file for SPL + DDR FW image

3.1.4.7 Encrypting and signing the SPL + DDR FW image

3.1.4.8 Creating the final CSF binary for SPL image

3.1.4.9 Creating the CSF description file for FIT image

3.1.4.10 Encrypting and signing the FIT image

3.1.4.11 Creating the final CSF binary for FIT image

Here the final images are “spl-imx8mm-trusty-dual-enc.bin” and “bootloader-imx8mm-trusty-dual-enc.img”.

Then after that it is mentioned to “See Section 3.1.2 to generate secure boot images and correctly closed the board”

Here, in section 3.1.2.2 Signing bootloader images

The signing process is doing for “spl-imx8mm-trusty-dual.bin” and “bootloader-imx8mm-trusty-dual.img”

So, from this we found that the encryption process is applied on one set of images i.e  “spl-imx8mm-trusty-dual-enc.bin” and “bootloader-imx8mm-trusty-dual-enc.img”.

And secure boot process is applied on another set of images i.e “spl-imx8mm-trusty-dual.bin” and “bootloader-imx8mm-trusty-dual.img”

So, here how to do the process? Do we need to implement secure boot first and after that implement encryption process on the same images?

6.) In  i.MX Android Security User's Guide

In secure boot section 3.1.2.2 Signing bootloader images
Execute the following commands to embed the CSF into the U-Boot image.

$ dd if=csf_spl.bin of=spl-imx8mm-trusty-dual.bin seek=$((0x34600)) bs=1 conv=notrunc

$ dd if=csf_fit.bin of=bootloader-imx8mm-trusty-dual.img seek=$((0x1020)) bs=1 conv=notrunc

Execute the following commands to insert the CSF into the U-Boot image.

$ dd if=csf_fit_fdt.bin of=bootloader-imx8mm-trusty-dual.img seek=$((0x3020)) bs=1 conv=notrunc

In encrypted boot section 3.1.4.13 Inserting CSF files and DEK blob

Insert csf_spl_sign_enc.bin in encrypted-spl.bin at the 0x35200 offset:

$ dd if=csf_spl_sign_enc.bin of=encrypted-spl-imx8mm-trusty-dual.bin seek=$((0x34600)) bs=1 conv=notrunc

Insert csf_fit_sign_enc.bin in encrypted-fit.bin at the 0x1020 offset:

$ dd if=csf_fit_sign_enc.bin of=encrypted-bootloader-imx8mm-trusty-dual.img seek=$((0x1020)) bs=1 conv=notrunc

Insert csf_fit_fdt.bin to encrypted-bootloader-imx8mm-trusty-dual.img at the 0x3020 offset:

$ dd if=csf_fit_fdt.bin of=encrypted-bootloader-imx8mm-trusty-dual.img seek=$((0x3020)) bs=1 conv=notrunc

Here, in both secure boot and encrypted boot the files are inserted at same offsets.

Is this correct?

7.) In  i.MX Android Security User's Guide

In section 3.1.2.4 Closing the chip

If we faced this error “Error: CSF header command not found” means how to solve it? Also are there any other things we need to verify before closing the chip?

8.) In  i.MX Android Security User's Guide

In section 3.1.2.1 Getting CST tool and keys configuration

It is mentioned “Files generated in ${CST}/keys/ and /release/crts/ are very important. If the SRK HASH value is fused to the chip and then changes the chip from open to close state, the board can only boot with images signed with these files.”

How to verify our images are properly signed or not and it will pass SRK verification or not before flashing the images?