KW45B41Z83AFTA MCU Configuration

Pre-Requisites:

SDK version: SDK_2_12_5_KW45B41Z-EVK

IDE: MCUXpresso IDE v11.9.0.2144

Query: The IRQ Handler (GPIOD_INT1_IRQHandler) is observed in the Button Task and the MCU is waiting for the interrupt trigger from the PTD1 pin, even though it is not configured.

Where is this Interrupt handler assignment taken care of in the code and how do I change it to GPIOB_INT1_IRQHandler??

Software Flow:

Pins Initialization (PTB4)

CLOCK_EnableClock(kCLOCK_PortB).

PORT_DisableEFTDetectInterrupts (PORTB, 0x08u); /* EFT detect interrupts configuration on PORTB*/

PORT_SetPinMux (S32K3_WKP_SRC_S32K3_WKP_PORT, S32K3_WKP_SRC_S32K3_WKP_PIN, kPORT_MuxAsGpio); /* PORTB3 (pin 1) is configured as PTB3 */

PORTB->PCR [3] = ((PORTB->PCR [3] & (~ (PORT_PCR_SRE_MASK | PORT_PCR_ODE_MASK | PORT_PCR_DSE_MASK)))

| PORT_PCR_SRE(PCR_SRE_sre1) /* Slew Rate Enable: Slow. */

| PORT_PCR_ODE(PCR_ODE_ode0) /* Open Drain Enable: Disables. */

| PORT_PCR_DSE(PCR_DSE_dse0)); /* Drive Strength Enable: Low. */

- APP_InitServices ():

Button handle: g_buttonHandle [1].

Button configuration:

- Pins default: 1 (high)

- Pins Direction: Input,

- Port Instance: 1 (PTB).

- Pin: 3 (PTB3).

Note: SW2(PTD1) and SW3(PTC6) configurations are removed.

- APP_ServiceInitLowpower ():

#define BOARD_WAKEUP_SOURCE_BUTTON0 (uint32_t) PM_WSID_PTB3_FALLING_EDGE